THREE PHASE 3-LEVEL T-Type ISOLATED BIDIRECTIONAL AC-DC CONVERTER FOR EV CHARGING

Fig.1 Three Phase 3-level T-TYPE Isolated Bidirectional Converter

PARAMETERS

| PARAMETERS | SPECIFICATIONS |
| --- | --- |
| Peak Power Rating | 6.6 kW |
| Switching Frequency | 120 kHz(AC to DC) |
| Input AC Voltages | 300-415 V, 50 Hz |
| DC Link Voltages | 900-1100 V |
| Battery Voltages | 600-800V |
| Primary side bridge duty ratio | 0.6 |
| Secondary side bridge duty ratio | 0.5 |
| Input AC Inductances | 0.8 mH |
| Turns Ratio | 55:80 |
| HF Transformer magnetizing inductance | 103.5 µH |
| Primary and Secondary Side Resonant Inductance | 78.8 µH |
| Primary and Secondary Side Resonant Capacitance | 32.145 nF |
| Split DC Link Capacitances | 660 µF |
| Output Split DC Capacitances | 440 µF |
| d-Current Controller | 5 + 1000/s |
| q-Current Controller | 5 + 1000/s |
| DC-link Voltage Controller | 0.1 + 10/s |
| Output Current Controller | 0.05 + 10/s |

MODULATION INDEX OF THE UPPER SWITCH

VAB1=0.707mam-1E

Where VAB1,MAX=input rms line voltage

ma=modulation index

m=number of voltage levels

E=voltage of each level

VAB1=415 V

m=3

E=VDC2=450 to 550V

This implies that

ma=0.53 to 0.65

SWITCHING FREQUENCY

- Switching frequency is selected between 100 kHz to 150 kHz.

- Most off-line AC/DC  applications require a switching frequency below  150  kHz for normal operation, and usually between 100 and 150 kHz as a rule of thumb.

- This is usually because conducting EMI testing starts at 150 kHz. Maintaining the switching frequency below the EMI test’s lower boundary helps the application pass the test.

- At high frequencies, the parameter of the resonant inductor and capacitor will be lower.

FILTER INDUCTOR

∆Irip_max=VDC2N-12fSL1

Where N is the number of voltage levels.

fS is switching frequency.

L1 is filter inductance.

VDC is DC Link Voltage.

∆Irip_max is the maximum ripple in the input line current.

∆Irip_max is taken as 10% of the rated peak current.

This implies that,

L1=882 μH

CAPACITOR CALCULATION

CO≥POUT4πfVO∆VO

- ∆VO is the Voltage ripple taken as 10% of DC Voltage.

- f is the line frequency.

This implies,

CDC≥129.68 μF

CO≥291.78 μF

We took,

CDC=292.5 μF

CO=390 μF

The rms ripple current passing through the capacitor is given by

Icaprms=Iphrms2M34π+3π-9M16cos2∅

For DC Link Capacitor,

Icaprms=5.9421 A

RESONANT CONVERTER PARAMETERS

The primary side and secondary side bridge voltages are given by

Vph=VDCjsin2πhDi+1-cos2πDi2hπ

Vsh=Vbst-sin2πhDo+13+sin2πh13+1-cos2πhDo+cos2πhDo+13-cos2πh13+jsin2πhDo2hπ

The final expressions for primary- and secondary-side bridge voltages can be drawn as

Vph=Vph,  Vsh=Vshejhγ,  γ=λ+ϕ,  λ=∠Vp1-∠Vs1

MODULATION SCHEME

FIG. : CLOSED-LOOP CONTROL BLOCK DIAGRAM

SIMULATION RESULTS

SWITCH SELECTION

FORMULA USED FOR CALCULATION OF CONDUCTION AND SWITCHING LOSS:

Ud0 = Vsd ( source to drain voltage) when drain current is zero

Io = Peak value of current

Ma =modulation index

Rdson = drain to source resistance

Rd= diode resistance

cos∅1= power factor

MOSFET switching losses:

DIODE switching losses:

Eon= turn on loss

Eoff=turn off loss

Vdc= input voltage

Ip= input current

Qrr= reverse recovery charge

| Switch Model Number | Manufacturer | Conduction Loss (IN Watt) | Switching Loss (in Watt) | Total Loss (in Watt) |
| --- | --- | --- | --- | --- |
| G3R40MT12K | GENESIC | 3.733 W | 8.3071 W | 12.041 W |
| IMZA120R040M1H | INFINEON | 4.357 W | 11.47 W | 15.825 W |
| IMZ1Z0R030M1H | INFINEON | 5.048 W | 16.5 W | 21.548 W |
| NVH4L030N120M3S | ONSEMI | 6.71 W | 11.835 W | 18.545 W |
| SCT4036KRHR | ROHM | 5.605 W | 11.63 | 17.235 |

SWITCH SELECTED: G3R40MT12K

GATE DRIVER

ACPL W346 is selected as it is suitable for SiC MOSFET and the recommended gate voltage of the switch.

RDSON=40 mΩ

Rg≥VCC-VEEIOLPEAK-RDSON

Where VEE= -5 V, VCC= 15 V, IOLPEAK=2.5 A.

Rg≥7.96 Ω

POWER SUPPLY GATE DRIVER

MGJ2D051505BSC with 2W power and output voltage of 15/-5V is selected.

DC LINK and OUTPUT CAPACITOR

CO≥POUT4πfVO∆VO

- ∆VO is the Voltage ripple taken as 10% of DC Voltage.

- f is the line frequency.

This implies,

CDC≥129.68 μF

CO≥291.78 μF

We took,

CDC=315 μF

CO=420 μF

Icap(rms)=Iph(rms)2M34π+3π-9M16cos2∅

For DC Link Capacitor,

Icap(rms)=5.9421 A

The capacitor taken is an Aluminium Electrolytic Capacitor with a value of 420uF and a current rating of 2.96 A with 420V. The series combination of 4 capacitors with 3 parallel lines will give the suitable capacitance.

The capacitor selected is EKHJ421VSN421MR51M.

Similarly, series combination of 3 capacitors and 3 parallel lines will give suitable capacitance for the output capacitor.

SNUBBER CAPACITOR

R53BI31505000K WITH 0.15uF.

RESONANT CAPACITANCE

Cr1=32.145nF, 1000V

R76UN233050H4J is selected for the purpose. Its rating is 33nF and 1000V. The combination of 2 series and 4 parallel lines for each terminal will give equivalent Cr1. The current rating of each capacitor is 4.6 A. So, it is suitable for the resonant current.

R76QI218050H4J is selected for the purpose. Its rating is 18nF and 1000V. The combination of 2 series and 4 parallel lines for each terminal will give equivalent Cr2. The current rating of each capacitor is 2.93 A. So, it is suitable for the resonant current.

HEAT SINK

- Power Loss in one switch is 12.041W.

- We will place 3 switches in one heat sink.

- Total power loss will be 3*12.041=36.123 W.

- Ambient temperature TA is taken as 35°C.

- Junction temperature TJ is taken as 150°C.

- Heat flow equation is PLOSS=TJ-TARJA

- RJA is the equivalent thermal resistance between the junction and the ambient.

- We must calculate RJH.

- Junction to heatsink thermal resistance RJH of MOSFET is 0.66 k/w.

- Equivalent RJA is 4.77535 K/W.

- Equivalent Junction to Heatsink thermal resistance RJH is calculated by the equivalent parallel of all the thermal resistances of MOSFET.

- It is calculated as 0.22 K/W.

- Equivalent Heatsink to Ambient Thermal Resistance RHA = 4.77535-0.22 = 4.55535 K/W.

- CR-201-75E has been selected. It has a thermal resistance of 2.6 K/W.

AC PFC INDUCTOR

- The calculated value of inductance is 882 μH.

- The current rating should be more than 1.5 times the peak line current.

- The selected inductor is B82747S6313N061.

- It is a power line choke with 3 winding on a single toroid core.

- The rating is 950uH, 31A, 250VAC.

Point to discuss:

- Loop Inductance of Gate Signal

- Heat sink

- DC Link Capacitor

SCHEMATIC DIAGRAMS

GATE DRIVER

- Reference from Texas Instruments.

- The track length between the Optocoupler and the Gate terminal is 15 mm.

- MCU generates a Gate Pulse of 0/3V.

- RC filter is used to filter the noise.

- An amplifier generates 0/5V and is compatible with an optocoupler.

- Optocoupler ACPL-W346 is used which is compatible with SiC-MOSFET.

- Gate Resistance for the forward path is taken as 15 ohms.

- 20 ohms resistance in series with antiparallel Schottky diode is taken. It will make a net backward gate resistance of 8.57 ohms.

- An antiparallel Zener diode is connected between the gate and the ground to avoid over-voltage across the gate.

- A DC/DC converter MGJ2D051505BSC with 2W, 5V to 15V/-5V supplies the optocoupler.

- A PCB consists of 6 gate driver circuits to provide gate pulses for each leg.

AC VOLTAGE SENSOR

- Differential Gain OPAMP Circuit is used to sense the phase voltages.

- Phase Voltage is fed to the resistor divider circuit to step down the voltage.

- A offset of 1.65V will make the voltage within the range of MCU.

- The gain of the sensor circuit is R3/R1.

- Here, R1=R2 and R3=R4.

- A PCB will consist of sensor circuit for all three phases.

- High voltage side is placed on MAIN BOARD.

- Low voltage side is placed on AC VOLTAGE SENSOR PCB.

DC VOLTAGE SENSOR

- R1 and R2 form the resistor divider circuit.

- R3, R4 and C1 forms RC filter circuit.

- An isolation amplifier AMC 1311 isolates the DC Link side from MCU ground.

- Opamp amplifies the signal to the required range of 0/3V.

- Signal is filtered using an RC circuit and fed into MCU.

- The cutoff frequency of filter 1 is 200kHz.

- The cutoff frequency of filter 2 is 1.7 kHz.

- The cutoff frequency of filter 3 is 15 kHz.

CURRENT SENSOR

- Hall effect-based Current Sensor is used.

- Sensor IC connects in series with phase track.

- The output voltage corresponds to the input current generated by the sensor.

- The voltage is filtered out and clamped up to make it in the range MCU.

AUXILIARY POWER SUPPLY

- RACM30-055K/277W is used to provide 230VAC to 5V with 30W rating.

- RAC10E-05SK/277 is used to provide 230VAC to 5V with 10W rating.

- This 5V will supply all the sensors with respect to GND_DSP.

- A non-isolated DC/DC converter will generate the 3.3V required by the sensors.

- The 5V terminal has isolated ground and will supply every gate driver's secondary circuit.

PCB FOOTPRINT

AC VOLTAGE SENSOR

DIMENSION: 50MM*36MM

TERMINAL DISTANCE: 5MM

DC VOLTAGE SENSOR

DIMENSION: 49MM*52MM

TERMINAL DISTANCE: 10MM

GATE DRIVER

DIMENSION: 189MM*58MM

TERMINAL DISTANCE: 26.5MM

DIMENSION: 81MM*58MM

Discussion Points:

- EMI filter placement on the Main Board.

- HF transformer terminal.

- RC filter in ADC terminal

- Simulation done in the LTSpice.

- Output is coming correct with the suitable RC filter.

- FRC cable placement.

- CCS coding.